#include "derivative.h"

#include <hidef.h>

// Define the value when no switches are pressed on Port H

#define PH_NONE 0xFF

// Global variables

unsigned char x; // Current Port H input

int shift;       // Direction of adjustment (+1 or -1)

int counter;     // Unused in this code

int mul;         // Multiplier for position adjustment

int DigitToSegment(int);

void MSDelay(unsigned int itime);

void disp(int);

char keypadScan();

const unsigned char keypad[4][4] =

{

'1','2','3','A',

'4','5','6','B',

'7','8','9','C',

'*','0','#','D'

};

unsigned char column,row;

// Keypad scanning function

void main(void) {

EnableInterrupts;

/* PWM initialization for servo motor control */

PWMPRCLK = 0x02; // Fbus / 2**2 = 24MHz / 2**2 = 6MHz

PWMSCLA = 3;     // 3MHz / 2*3 = 1MHz

// Select PP(bit register)

PWMCLK = 0x20; // ClockSA for chan 4

PWMPOL = 0x20; // high then low for polarity

PWMCAE = 0x00; // left aligned

PWMCTL = 0x40; // Concat 4 & 5

PWMPER45 = 20000; // 1MHz / 20000 = 50Hz for motor

PWMDTY45 = 1500;  // 1500 / 20000 * 20ms = 1.5ms (neutral)

DDRA = 0x0F;                           //MAKE ROWS INPUT AND COLUMNS OUTPUT

DDRB = 0xFF; // Enable output ports for debugging

DDRP = 0xFF;

DDRH = 0x00;

PWME = 0x30; // Enable PWM channels 4 and 5

// Initialize control variables

shift = 1; // Default to positive direction

mul = 80;  // Adjustment multiplier

for (;;) {

x = keypadScan();

disp(PWMDTY45);

MSDelay(10);

if (x == 0) {

continue;

}

// makes rotation clockwise

if (x == '8') {

shift = -1;

MSDelay(20);

}

// makes rotation counter clockwise

if(x == '9') {

shift = 1;

MSDelay(20);

}

//Make Rotation Speed faster

if (x == '5') {

mul = mul + 10;

MSDelay(10);

}

//Make rotation speed slower

if (x == '4') {

mul = mul - 10;

MSDelay(10);

}

if (mul > 200) {

mul = 200;

} else if (mul < 20) {

mul = 20;

}

// rotate to 180 degrees

if (x == 'A') {

while (PWMDTY45 < 2500) {

PWMDTY45 += 83;

MSDelay(20);

}

}

// rotate to 90 degrees

if (x == '3') {

if (PWMDTY45 > 1500) {

while (PWMDTY45 > 1500) {

PWMDTY45 -= 83;

MSDelay(20);

}

} else if (PWMDTY45 < 1500) {

while (PWMDTY45 < 1500) {

PWMDTY45 += 83;

MSDelay(20);

}

}

}

// rotate to 0 degrees

if (x == '2') {

while (PWMDTY45 > 500) {

PWMDTY45 -= 83;

MSDelay(20);

}

}

if (x == '1') {

// Can change mul for finer control

PWMDTY45 += shift * mul;

// Clamp duty cycle to valid servo range

// 150 for 90 deg, 120 for 45 deg

if (PWMDTY45 > 2500) {

PWMDTY45 = 2500;

} else if (PWMDTY45 < 500) {

PWMDTY45 = 500;

}

MSDelay(20);

}

if (x != '1' && x != '4' && x != '5') {

do {

PORTA = PORTA | 0x0F;

row = PORTA & 0xF0;

MSDelay(10);

}while (row != 0x00);

}

if (PWMDTY45 > 2500) {

PWMDTY45 = 2500;

} else if (PWMDTY45 < 500) {

PWMDTY45 = 500;

}

// Delay to control loop speed and allow display multiplexing

MSDelay(20);

}

}

int DigitToSegment(int digit) {

switch (digit) {

case 0:

return 0x3F;

case 1:

return 0x06;

case 2:

return 0x5B;

case 3:

return 0x4F;

case 4:

return 0x66;

case 5:

return 0x6D;

case 6:

return 0x7D;

case 7:

return 0x07;

case 8:

return 0x7F;

case 9:

return 0x6F;

default:

return 0x00;

}

}

void MSDelay(unsigned int itime) {

unsigned int i;

unsigned int j;

for (i = 0; i < itime; i++)

for (j = 0; j < 4000; j++)

;

}

void disp(int num) {

PORTB = DigitToSegment(num / 1000);

PTP = 0x0E;

MSDelay(2);

PORTB = DigitToSegment((num / 100) % 10);

PTP = 0x0D;

MSDelay(2);

PORTB = DigitToSegment((num / 10) % 10);

PTP = 0x0B;

MSDelay(2);

PORTB = DigitToSegment(num % 10);

PTP = 0x07;

}

char keypadScan() {

while (1) {

//OPEN do1

PORTA = PORTA | 0x0F;            //COLUMNS SET HIGH

row = PORTA & 0xF0;              //READ ROWS

if (row == 0x00) {

return 0;

}

do{                                 //OPEN do2

do{                              //OPEN do3

MSDelay(1);                   //WAIT

row = PORTA & 0xF0;           //READ ROWS

}while(row == 0x00);             //CHECK FOR KEY PRESS //CLOSE do3

MSDelay(15);                     //WAIT FOR DEBOUNCE

row = PORTA & 0xF0;

}while(row == 0x00);                //FALSE KEY PRESS //CLOSE do2

while(1){                           //OPEN while(1)

PORTA &= 0xF0;                   //CLEAR COLUMN

PORTA |= 0x01;                   //COLUMN 0 SET HIGH

row = PORTA & 0xF0;              //READ ROWS

if(row != 0x00){                 //KEY IS IN COLUMN 0

column = 0;

break;                        //BREAK OUT OF while(1)

}

PORTA &= 0xF0;                   //CLEAR COLUMN

PORTA |= 0x02;                   //COLUMN 1 SET HIGH

row = PORTA & 0xF0;              //READ ROWS

if(row != 0x00){                 //KEY IS IN COLUMN 1

column = 1;

break;                        //BREAK OUT OF while(1)

}

PORTA &= 0xF0;                   //CLEAR COLUMN

PORTA |= 0x04;                   //COLUMN 2 SET HIGH

row = PORTA & 0xF0;              //READ ROWS

if(row != 0x00){                 //KEY IS IN COLUMN 2

column = 2;

break;                        //BREAK OUT OF while(1)

}

PORTA &= 0xF0;                   //CLEAR COLUMN

PORTA |= 0x08;                   //COLUMN 3 SET HIGH

row = PORTA & 0xF0;              //READ ROWS

if(row != 0x00){                 //KEY IS IN COLUMN 3

column = 3;

break;                        //BREAK OUT OF while(1)

}

row = 0;                         //KEY NOT FOUND

break;                              //step out of while(1) loop to not get stuck

}

if(row == 0x10){

return keypad[0][column];

}

else if(row == 0x20){

return keypad[1][column];

}

else if(row == 0x40){

return keypad[2][column];

}

else if(row == 0x80){

return keypad[3][column];

}

return 0;

}

}